USER GUIDE: C TO ASM AND ASM TO C INTERFACE AND CODING GUIDELINES

Document history:

| S. No | Ver.No. | Author | Section(s) Affected | Change(s) made | Approved Date | Approved by |
| --- | --- | --- | --- | --- | --- | --- |
| 1 | 1.0 | Vinitha | All | All |  |  |

Table of Contents

1	INTRODUCTION	3

2	Memory Section Definitions	3

2.1	Required Sections in LDF	3

3	Register Management	3

4	Declaration in C and ASM	4

4.1	C Declarations	4

4.2	ASM Declarations	4

5	Naming Conventions	4

5.1	General Rules	4

5.2	Argument Passing	5

5.3	Register Assignments	5

6	steps to implement an C and ASM code	5

7	Example ADD PROGRAM	6

8	CHALLENGES FACED	7

9	Conclusion	7

10	REFERENCES	8

# INTRODUCTION

This guide is designed for developers who need to interface C and Assembly code in embedded systems, particularly those using the SHARC architecture. It covers essential topics such as register management, function calling conventions, argument passing, and memory section definitions. Understanding these concepts will facilitate effective communication between C and ASM, leading to optimized performance in embedded applications.

# Memory Section Definitions

Proper memory section definitions are necessary for organizing code and data in embedded applications.

## Required Sections in LDF

- Seg_pmda: Program memory data section, declared as 32-bit wide.

- Seg_pmco: Program memory code section, declared as 48-bit wide.

- Seg_swco: Program memory code section.

- Seg_dmda: Data memory section, declared as 32-bit wide.

- Seg_stak: C-Runtime Stack section declaration.

- Seg_heap: C-Heap section declaration.

# Register Management /////

Registers SHARC is a load-store machine. It means all the data needs to be loaded into registers before computation. There are a lot of registers, but here I am only going to outline ones that are commonly used in programming: Data Registers / Register File The register file has 16 registers, named R0 to R15, and F0 to F15.

The R version is used in fixed-point calculations, and the F version is used in floating-point calculations. Unlike on common RISC architectures, F registers are not separate registers, they are just alias for R registers and share the same physical register.

For example, setting R5 to 0x12345678 will also set F5 to 0x12345678 (whatever that means in IEEE754). Note on SHARC these registers are actually 40bit wide, so you could get a larger range or higher precision when doing the calculation. There is also a hardware mode allowing you to limit the floating-point operation to only 32bit.

DAG Registers Unlike common RISC machines, SHARC cannot address memory with data registers. This means you cannot simply load an address into some data register and access the memory by doing a load with that register. Instead, separate DAG (Data Address Generator) registers are provided for these purposes.

There are 4 sets of DAG registers:

- I0-15, index registers

- M0-15, modify registers

- B0-15, base registers

- L0-15, length registers.

As always, you can find the full definition of all registers in the PRM.

As said there will be a two DAG registers (1,2) which will control one set of data memory, it will store only (R0 – R7) and (I0-I7) and other will used to access other set of program memory, it is used to store (R8-R15) and (I8-I15),same will be applicable to modifier registers.

Example:

R3 = dm (i2, m6);   ///dm-

R9 = pm (i9, m14); ///pm-

/// 4 Operators supported

/// 5 Loops

/// 6 conditional

# Declaration in C and ASM

Proper variable declaration is crucial for ensuring data types are correctly interpreted by both C and ASM. In CrossCore Embedded Studio (CCES), include “asm_sprt.h” in the assembly file, use entry at the start and exit at the end of the function, and ensure the result is returned in the R0 register for proper integration with C code.

## C Declarations

In C, the data type must be explicitly declared to avoid type-related errors. For example, you would declare an integer as "int example;" and a float as "float exampleFloat;".

## ASM Declarations

In ASM, variables are declared using directives. For instance, you might declare a variable as ".var temp;" or an array with ".var buffer[10];". Generally to access globally must include “.extern” keyword      ////.global

# Naming Conventions

Consistent naming conventions help maintain clarity when interfacing between C and ASM.

## General Rules

When using C symbols in ASM, prefix them with an underscore to distinguish them from ASM symbols. For example, in ASM you would declare a C variable as ".var _temp;". In C, you would declare an ASM variable with "extern int temp;".

## Function call ////

## Argument Passing

Arguments are passed between C and ASM using registers and the stack, which is crucial for function execution.

## // Register Assignments

The first three arguments are passed using specific registers:

- First Argument: r4

- Second Argument: r8

- Third Argument: r12 Any additional arguments are passed via the stack and can be accessed using the reads(n)- [n=1, 2,..] instruction. The return value of a function is typically stored in r0.

Code Snippet:

//example

# steps to implement an C and ASM code //// edit

Here is the step-by-step explanation for integrating C and assembly code in a project within CrossCore Embedded Studio (CCES):

- Step 1:
	Create a C file where you will include the necessary declarations and initializations. Within the C file, declare the function prototype for the assembly function you will be writing later. You can also call this function in your C code to test its functionality.

- Step2:
	For the function you declared in the C file, create a corresponding assembly file within the same project. In this assembly file, write the assembly code for the function. Ensure that only the code relevant to that function is written in the assembly file.

- Step 3:
	In the assembly file, include the appropriate memory sections to allocate space for the code and data. Use the .extern directive to declare any global variables that need to be accessed in the assembly code. Also, ensure the function name in the assembly file starts and ends with a period (.) as a naming convention.

- Step4:
	Within the assembly function, begin the function definition. When the function is called from the C file, the result will automatically be returned in the R0 register, which is the standard register for return values in many ARM-based systems.

- Step5:
	You can verify that the function works correctly by using CCES's register browser to inspect the values in the R0 register, where the result will be stored. Alternatively, you can use printf in the C file to print the result and check if it matches the expected output.

By following these steps, you can successfully integrate assembly and C code in CCES, allowing you to write performance-critical functions in assembly while still leveraging the power of C for higher-level operations.

# Conclusion

Combining C and Assembly in embedded systems, especially with the SHARC architecture, helps create efficient and optimized applications. Understanding how to manage registers, pass arguments, call functions, and define memory sections allows developers to seamlessly integrate high-level and low-level code. Using Assembly provides the advantage of optimized performance with significant reductions in cycle counts, making programs faster and more effective for critical tasks.

# REFERENCES

[1] https://zephray.me/post/sharc_tutorial_1/

[2] Getting Started with SHARC+ Assembly Programming, part 2: Branch and ...

[3] Loop Sequencer - Q&A - SHARC Processors - EngineerZone