USER GUIDE: C TO ASM AND ASM TO C INTERFACE AND CODING GUIDELINES

Document history:

| S. No | Ver.No. | Author | Section(s) Affected | Change(s) made | Approved Date | Approved by |
| --- | --- | --- | --- | --- | --- | --- |
| 1 | 1.0 | Vinitha | All | All |  |  |

Table of Contents

1	INTRODUCTION	3

2	Memory Section Definitions	3

3	Register Management	3

4	Operators supported	7

1.1	ALU Fixed Point Computations	7

5	Loop	8

6	Declaration in C and ASM	11

1.2	C Declarations	11

1.3	ASM Declarations	11

7	Naming Conventions	11

1.4	General Rules	11

8	FUNCTIONS	11

1.5	7.1 Function Call: C to Assembly	11

1.5.1	Steps:	11

1.6	7.2     Function Call: Assembly to C	12

7.2.1 Steps:	12

1.7	12

1.8	7.3    Function Call: Assembly to Assembly	12

7.3.1 Steps:	12

1.9	7.4 Argument passing	13

7.4.1 Assignments:	13

7.4.2 Additional Arguments:	13

9	steps to implement an C and ASM code //// edit	14

10	Conclusion	15

11	REFERENCES	16

# INTRODUCTION

This guide is designed for developers who need to interface C and Assembly code in embedded systems, particularly those using the SHARC architecture. It covers essential topics such as register management, function calling conventions, argument passing, and memory section definitions. Understanding these concepts will facilitate effective communication between C and ASM, leading to optimized performance in embedded applications.

# Memory Section Definitions

Proper memory section definitions are necessary for organizing code and data in embedded applications.

Required Sections in LDF

- Seg_pmda: Program memory data section, declared as 32-bit wide.

- Seg_pmco: Program memory code section, declared as 48-bit wide.

- Seg_swco: Program memory code section.

- Seg_dmda: Data memory section, declared as 32-bit wide.

- Seg_stak: C-Runtime Stack section declaration.

- Seg_heap: C-Heap section declaration.

# Register Management

The table below explains the types of registers available and its function.

| Register Type | Register Name | Description | Example Usage |
| --- | --- | --- | --- |
| Data Registers (R) | R0 to R15 | Used for fixed-point calculations. R and F registers share the same physical space. | R3 = dm(i2, m6) (fixed-point operation) |
| Floating-Point Registers (F) | F0 to F15 | Used for floating-point calculations, alias for the R registers. | R5 = 0x12345678 (same value in F5) |
| DAG Registers (Index) | I0 to I15 | Stores index values for addressing memory. | Not directly accessed, used in memory ops |
| DAG Registers (Modify) | M0 to M15 | Stores modify values for memory addressing. | Used alongside index and base registers |
| DAG Registers (Base) | B0 to B15 | Stores base addresses for memory addressing. | Used in memory access operations |
| DAG Registers (Length) | L0 to L15 | Stores length values for memory addressing. | Used for memory block operations |

Memory Set Access

| Memory Type | DAG Register Pair | Data Registers (R) | Index Registers (I) | Modify Registers (M) | Base Registers (B) | Length Registers (L) | Example Usage |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Data Memory (dm) | DAG Register Pair 1 (I0-I7, M0-M7) | R0 - R7 | I0 to I7 | M0 to M7 | B0 to B7 | L0 to L7 | R3 = dm(i2, m6) (Access data memory) |
| Program Memory (pm) | DAG Register Pair 2 (I8-I15, M8-M15) | R8 - R15 | I8 to I15 | M8 to M15 | B8 to B15 | L8 to L15 | R9 = pm(i9, m14) (Access program memory) |

| Register Type | Registers | Function | Example |
| --- | --- | --- | --- |
| Data Registers (Dreg) | r0 - r15 | Processing element X (PEx) register file locations, fixed-point | r0 = 5; (Assigns a fixed-point value of 5 to r0) |
|  | f0 - f15 | PEx register file locations, floating-point | f0 = 3.14f; (Assigns a floating-point value of 3.14 to f0) |
|  | r0 - r15, f0 - f15 | PEx register file locations, fixed-point or floating-point | f0 = r0 + 2.5f; (Adds fixed-point value in r0 to a float) |
|  | r1:0 - r15:14, f1:0 - f15:14 | PEx 64-bit register file locations, fixed-point or floating-point | r1:0 = r2:1; (Moves data from 64-bit register r2:1 to r1:0) |
| Complementary Data Registers (CDreg) | s0 - s15, sf0 - sf15 | Processing element Y (PEy) register file locations, fixed-point and floating-point | sf0 = 1.5f; (Assigns a floating-point value to sf0) |
| System Registers (Sysreg) | astat, astatx, astaty | PE, PEx, PEy arithmetic status flags and bit test flag | astat = 0; (Clears the arithmetic status flag astat) |
|  | flags | Flag pins input/output state | flags = 1; (Sets flag state to active) |
|  | imask | Interrupt mask | imask = 0xFF; (Enables interrupts for all bits in the mask) |
|  | imaskp | Interrupt mask pointer (for nesting) | imaskp = &imask; (Points imaskp to imask) |
|  | irptl | Interrupt latch | irptl = 0x01; (Sets interrupt latch for specific IRQ) |
|  | mmask | Mode mask | mmask = 0x03; (Configures mode mask to mode 3) |
|  | mode1 | Mode 1 control and status | mode1 = 1; (Sets the mode 1 control flag) |
|  | mode1stk | Mode 1 stack (top-most entry) | mode1stk = 0x2000; (Sets stack entry for mode 1) |
|  | mode2 | Mode 2 control and status | mode2 = 0x04; (Sets mode 2 control) |
|  | stky, stkyx, stkyy | PE, PEx, PEy sticky status flags and stack status flags | stky = 1; (Sets sticky flag for PE) |
|  | ustat1, ustat2, ustat3, ustat4 | User status 1, 2, 3, and 4 | ustat1 = 0x03; (Sets user status flag) |
| Index Registers (Ia) | i0 - i7 | Index registers, Data Address Generator 1 (DAG1) | i0 = 0x1000; (Sets address in i0 for DAG1) |
| Modifier Registers (Mb) | m0 - m7 | Modify registers, DAG1 | m0 = 0x04; (Sets modifier value in m0 for DAG1) |
| Base Registers (Ba) | b0 - b7 | Base registers, DAG1 | b0 = 0x2000; (Sets base address in b0) |
| Index Registers (Ic) | i8 - i15 | Index registers, DAG2 | i8 = 0x3000; (Sets index address in i8 for DAG2) |
| Modifier Registers (Md) | m8 - m15 | Modify registers, DAG2 | m8 = 0x08; (Sets modifier value in m8 for DAG2) |
| Base Registers (Bc) | b8 - b15 | Base registers, DAG2 | b8 = 0x4000; (Sets base address in b8 for DAG2) |
| Universal Register (Ureg) | l0 - l7, l8 - l15 | Length registers, DAG1 and DAG2 | l0 = 0x10; (Sets loop length in l0) |
|  | px, px1, px2 | PMD-DMD bus exchange (64-bit and 32-bit) | px1 = 0x8000; (Sets 32-bit PMD-DMD bus exchange value) |
|  | pc, pcstk, pcstkp | Program counter, top of PC stack, PC stack pointer | pc = 0x0010; (Sets program counter to address 0x0010) |
|  | faddr, daddr, laddr | Fetch, decode, and loop termination addresses | faddr = 0x0100; (Sets fetch address in faddr) |
|  | curlcntr, lcntr | Current and loop counters | curlcntr = 10; (Sets loop counter curlcntr to 10) |
|  | tperiod, tcount | Timer period and counter | tperiod = 1000; (Sets timer period in tperiod) |
|  | emuclk, emuclk2 | Emulator clocks | emuclk = 1; (Sets emulator clock state) |
| Universal Register (Ureg) Additional Classes | f1:0 - f15:14, sf1:0 - sf15:14 | Double-precision floating-point data registers for PEx and PEy | f1:0 = 3.14159; (Sets double-precision value for PEx) |
|  | Same as UREG but omits DAG1-specific registers | Same as UREG but without DAG1-specific registers | UREGXDAG1 = 0x1234; (Sets register without DAG1-specific regs) |
|  | Same as UREGDBL | Same as UREGDBL but without DAG1-specific registers | UREGXDAG1DBL = 0x5678; (Sets double-precision without DAG1) |
|  | Same as UREG but omits DAG2-specific registers | Same as UREG but without DAG2-specific registers | UREGXDAG2 = 0x9ABC; (Sets register without DAG2-specific regs) |
|  | Same as UREGDBL | Same as UREGDBL but without DAG2-specific registers | UREGXDAG2DBL = 0xDEF0; (Sets double-precision without DAG2) |

# Operators supported

NOTE: Below mentioned operators are properly worked only in ADSP-SC5xx and ADSP-215xx Processors. If you try with other SHARC processors, refer to that Program reference Manual.

## ALU Fixed Point Computations

| Arithmetic Operators |  |
| --- | --- |
| Syntax | Description |
| RN = RX + RY | Adds the fixed-point fields in registers Rx and Ry. The result is placed in the fixed-point field in register Rn. |
| RN = RX - RY | Subtracts the fixed-point fields in registers Rx and Ry. The result is placed in the fixed-point field in register Rn. |
| RN = RX + RY + ci | Adds with carry (AC from ASTAT) the fixed-point fields in registers Rx and Ry. The result is placed in the fixed-point register Rn |
| RN = RX – RY + ci – 1 | Subtracts with borrow (AC - 1 from ASTAT) the fixed-point field in register Ry from the fixed-point field in register Rx. The result is placed in the fixed-point field in register Rn. |
| RN = RX + ci | Adds the fixed-point field in register Rx with the carry flag from the ASTAT register (AC). The result is placed in the fixed-point field in register Rn |
| RN = RX + ci – 1 | Adds the fixed-point field in register Rx with the borrow from the ASTAT register (AC - 1). The result is placed in the fixed-point field in register Rn.) |
| RN = –RX | Negates the fixed-point operand in Rx by two's-complement. The result is placed in the fixed-point field in register Rn |

| Logical Operators |  |
| --- | --- |
| Syntax | Description |
| RN = RX and RY | Logically ANDs the fixed-point operands in Rx and Ry. The result is placed in the fixed-point field in Rn. |
| RN = RX or RY | Logically ORs the fixed-point operands in Rx and Ry. The result is placed in the fixed-point field in Rn |
| RN = RX xor RY | Logically XORs the fixed-point operands in Rx and Ry. The result is placed in the fixed-point field in Rn |
| RN = not RX | Logically NOTs the fixed-point operands in Rx. The result is placed in the fixed-point field in Rn. |

| Relational Operators |  |
| --- | --- |
| Syntax | Description |
| comp (RX, RY) | Compares the signed fixed-point field in register Rx with the fixed-point field in register Ry. Sets the AZ flag if the two operands are equal, and the AN flag if the operand in register Rx is smaller than the operand in register Ry. The MSB bit of CACC is set if the X operand is greater than the Y operand |
| compu (RX, RY) | Compares the unsigned fixed-point field in register Rx with the fixed-point field in register Ry. Sets the AZ flag if the two operands are equal, and the AN flag if the operand in register Rx is smaller than the operand in register Ry. The MSB bit of CACC is set if the X operand is greater than the Y operand |
| EQ | Based on compare operation, if equal it will jump to assigned label. |
| NE | Based on compare operation, if not equal it will jump to assigned label. |
| GT | Based on compare operation, if greater than it will jump to assigned label. |
| LT | Based on compare operation, if less than it will jump to assigned label. |
| GE | Based on compare operation, if greater than or equal it will jump to assigned label. |
| LE | Based on compare operation, if less than or equal it will jump to assigned label. |
| RN = min (RX, RY) | Returns the smaller of the two fixed-point operands in Rx and Ry. The result is placed in the fixed-point field in register Rn. |
| RN = max (RX, RY) | Returns the larger of the two fixed-point operands in Rx and Ry. The result is placed in the fixed-point field in register Rn. |

# Loop

| REGISTER | DESCRIPTION |
| --- | --- |
| LCNTR | Loop counter (LCNTR) register has top level entry to loop counter stack. |
| CURLCNTR | Current loop counter (CURLCNTR) indicates the current loop status. |

Example:

Code snippet :

| Syntax | C file | ASM file |
| --- | --- | --- |
| LCNTR =<data16>, DO (PC, <reladdr24>) UNTIL LCE; | #include <stdio.h> void add_arrays(int *,int *, int*,int); int a[5]= {1,2,3,4,5}; int b[5] = {6,7,8,9,10}; int sum[5]= {0}; int main() { add_arrays(a,b,sum,5); for(int i=0;i<5;i++) { printf("%d \n", sum[i]); } return 0; } | #include <asm_sprt.h> .global add_arrays.; .section/pm seg_pmco; .type add_arrays., STT_FUNC; add_arrays.: entry; i1 = r4; i2 = r8; i3 = r12; r1=reads(1); lcntr = r1; do (pc, .P1L9) until lce; r5 = dm(i1, m6); r6 = dm(i2, m6); r7 = r5 + r6; .P1L9:dm(i3, m6) = r7; exit; .add_arrays..end: |
| LCNTR = <data16>, DO <addr24> UNTIL LCE; |  | #include <asm_sprt.h> .global add_arrays.; .section/pm seg_pmco; .type add_arrays., STT_FUNC; add_arrays.: entry; i1 = r4; i2 = r8; i3 = r12; r1=reads(1); lcntr = r1; do loop_start until lce; r5 = dm(i1, m6); r6 = dm(i2, m6); r7 = r5 + r6; loop_start:dm(i3, m6) = r7; exit; .add_arrays..end: |
| CURLCNTR |  | #include <asm_sprt.h> .global add_arrays.; .section/pm seg_pmco; .type add_arrays., STT_FUNC; add_arrays.: entry; i1 = r4; i2 = r8; i3 = r12; r1=reads(1); lcntr = r1; do loop_start until lce; r5 = dm(i1, m6); r6 = dm(i2, m6); r7 = r5 + r6; r2=CURLCNTR; //to know about the cuurent loop status loop_start:dm(i3, m6) = r7; exit; .add_arrays..end: |

/// 6 conditional statment

# Declaration in C and ASM

Proper variable declaration is crucial for ensuring data types are correctly interpreted by both C and ASM. In CrossCore Embedded Studio (CCES), include “asm_sprt.h” in the assembly file, use entry at the start and exit at the end of the function, and ensure the result is returned in the R0 register for proper integration with C code.

## C Declarations

In C, the data type must be explicitly declared to avoid type-related errors. For example, you would declare an integer as "int example;" and a float as "float exampleFloat;".

## ASM Declarations

In ASM, variables are declared using directives. For instance, you might declare a variable as ".var temp;" or an array with ".var buffer[10];". Generally to access globally must include “.extern” keyword or “. global”.

# Headers and directives:

At the Start include “asm_sprt.h”  to support the entry and exit in the asm code. Use .type directive directs the assembler to change the symbol type of an object. This directive may appear in the compiler-generated assembly source file(.s).

Syntax:

.TYPE symbolName, symbolType;

symbolName is the name of the object to which the symbolType is applied.

symbolType is an ELF symbol type STT_*. ValidELF symbol types are listed in the ELF.h header file. By default, a label has an STT_FUNC symbol type, and a variable or buffer name defined in a storage directive has an STT_OBJECT symbol type.

# Naming Conventions

Consistent naming conventions help maintain clarity when interfacing between C and ASM.

## General Rules

When using C symbols in ASM, prefix them with an underscore to distinguish them from ASM symbols. For example, in ASM you would declare a C variable as ".var _temp;". In C, you would declare an ASM variable with "extern int temp;".

# FUNCTIONS

## 7.1 Function Call: C to Assembly

### Steps:

- Declare the ASM function in C using extern.

- In the ASM file, declare the function as global using .global.

- Implement the function in the .asm file and use rts to return.

#### Example:

#### C File:

extern void Add(void); 	/* Declare the ASM function */
int main () {
    Add ();          		/* Call the ASM function */
    return 0;
}

ASM File:

.global Add;         /* Declare Add globally */
.section/pm seg_pmco;
Add:
    r4 = 5;
    r8 = 15;
    r0 = r4 + r8;       	/* Perform addition */
    rts;                	/* Return to caller */

## 7.2     Function Call: Assembly to C

### 7.2.1 Steps:

- Declare the C function globally in the ASM file using .extern.

- Use the call instruction to invoke the C function.

- Ensure all necessary registers are saved and restored in ASM.

#### Example:

C File:

int Myfunc(int a, int b) {
    return a + b;
}

ASM File:

.global Myfunc;      /* Declare Myfunc globally */
.extern Myfunc;      /* Access the C function */
.section/pm seg_pmco;
call_func:
    r4 = 10;         /* First argument */
    r8 = 20;         /* Second argument */
    call Myfunc;     /* Call the C function */
    dm(result) = r0; /* Store the return value */
    rts;

## 7.3    Function Call: Assembly to Assembly

### 7.3.1 Steps:

- Declare both functions as global in the ASM file using. global.

- Use the call instructions for invoking the subroutine.

- Use rts to return from the subroutine.

#### Example:

#### ASM File:

.global Add;
.global Multiply;
.section/pm seg_pmco;
/* Add Function */
Add:
    r4 = 10;
    r8 = 20;
    r0 = r4 + r8;      /* Perform addition */
    rts;
/* Multiply Function */
Multiply:
    call Add;          /* Call Add function */
    r0 = r0 * 2;       /* Multiply result by 2 */
    rts;

## 7.4 Argument passing

Arguments are passed between C and ASM using registers and the stack, which is crucial for function execution.

### 7.4.1 Assignments:

The first three arguments are passed using specific registers:

- First Argument: r4

- Second Argument: r8

- Third Argument: r12

### 7.4.2 Additional Arguments:

When passing more than three arguments to a function in SHARC+ assembly, the additional arguments are placed on the stack. These arguments can be accessed using the reads(n) instruction, where n specifies the position of the argument on the stack.

- For example:

- reads (1) holds the value of the fourth argument.

- reads (2) holds the value of the fifth argument, and so on.

- The return value of a function is typically stored in the R0 register.

Example:

An example of passing arguments to a function is provided in the code snippet below.

| C File | ASM file |
| --- | --- |
| #include <stdio.h> int32_t function (int32_t, int32_t, int32_t, int32_t, int32_t, int32_t); int main() { //Implement a equation, result = (m + n) + (a - b) - (c - d) int32_t result=0; result= function(2,4,6,8,10,12); printf("%d \n",result); return 0; } | #include <asm_sprt.h> section/pm seg_pmco; .global function.; .type function., STT_FUNC;. function.: entry; r1=r4; r2=r8; r3=r12; r4=reads(1); r5=reads(2); r6=reads(3); r13 = r1+r2; r14 = r3-r4; r15 =r5-r6; r0=0; r0=r0+r13; r0=r0+r14; r0=r0-r15; exit; .function..end: |

# steps to implement an C and ASM code //// edit

Here is the step-by-step explanation for integrating C and assembly code in a project within CrossCore Embedded Studio (CCES):

- Step 1: 
  Create a C file to include the necessary functions, variable declarations, and initializations. Within the C file, declare the prototype for the assembly function you will write later. You can also call this function in your C code to test its functionality.

- Step2: 
  For the function you declared in the C file, create a corresponding assembly file within the same project by adding new file and name it with .asm extension. In this assembly file, write the assembly code for the function. Ensure that only the code relevant to that function is written in the assembly file.

- Step 3: 
  In the assembly file, include the appropriate memory sections to allocate space for the code and data. Use the .extern (an external linkage specifier) directive to declare any global variables that need to be accessed in the assembly code. Also, ensure the function name in the assembly file starts and ends with a period (.) as a naming convention.

| In C | In ASM |
| --- | --- |
| Test(A,B); | .section/pm seg_pmco; // Included in the program memory .extern Test.; .type Test., STT_FUNC; // Start Test Function with type of test Test.: .Test..END // Test Function End |

- Step4: 
  Within the assembly function, begin the function definition. When the function is called from the C file, the result will automatically be returned in the R0 register, which is the standard register for return values in many ARM-based systems.

In C In ASM

.section/pm seg_pmco; Int C= Test(A,B); .extern Test.;

.type Test., STT_FUNC;

Test.:

R0=R4+R8 // R0 holds the returned value of the sum.

.Test..END

- Step5: 
  You can verify that the function works correctly by using CCES's register browser to inspect the values in the R0 register, where the result will be stored while in the state of debugging. Alternatively, you can use the printing function in the C file to print the result and check if it matches the expected output.

- Steps to Obtain Register Browser in the Interface

  - 1. CCES->Quick access -> Type -> Register Browser

  - 2.CCES->Window->Show view-> Others->Debug-> Register Browser.

By following these steps, you can successfully integrate assembly and C code in CCES, allowing you to write performance-critical functions in assembly while still leveraging the power of C for higher-level operations.

# Conclusion

Combining C and Assembly in embedded systems, especially with the SHARC architecture, helps create efficient and optimized applications. Understanding how to manage registers, pass arguments, call functions, and define memory sections allows developers to seamlessly integrate high-level and low-level code. Using Assembly provides the advantage of optimized performance with significant reductions in cycle counts, making programs faster and more effective for critical tasks.

# REFERENCES

[1] https://zephray.me/post/sharc_tutorial_1/

[2] Getting Started with SHARC+ Assembly Programming, part 2: Branch and ...

[3] Loop Sequencer - Q&A - SHARC Processors - EngineerZone